HYDROPOWER LINE DIFFUSER SYSTEM INSTALLATIONS

- Lake Lillinonah, Shepaug Dam

- Hydropower reservoir

- Located near Danbury, Connecticut

- Reservoir:  74,000 ac-ft, 1,870 acres

- Hydropower:  one 43 MW hydroturbine unit; 5,900 cfs turbine flow

- Purpose for Aeration:  Maintain 5 mg/L of dissolved oxygen in reservoir releases

- Customer:  Northeast Generation Services

- Aeration System:  Line Diffuser

- - 13,000-gallon bulk liquid oxygen storage facility

- - Three 3,720-foot line diffusers arranged in the intake channel and forebay

- - Total of 11,160 feet of line diffuser deployed in reservoir

- - 1,500 scfm or 90-tons/day oxygen system capacity

- Installed by Mobley Engineering, Inc.: 2006

- Tims Ford Reservoir

- Hydropower reservoir

- Located near Winchester, Tennessee

- Reservoir:  600,000 ac-ft; 12,400 acres

- Hydropower:  one 45-MW hydroturbine units; 4,500 cfs turbine flow

- Purpose for Aeration:  Maintain by 4 mg/L of dissolved oxygen in hydroturbine releases

- Customer: Tennessee Valley Authority, Lake Improvement Plan II

- Aeration System: Line Diffusers

- - Four line diffusers arranged in the forebay along old riverbed

- - Total of 15,500 feet of line diffuser deployed in reservoir

- Installed by TVA:  2005

- Lake Wallenpaupack

- Hydropower reservoir

- Located near Hawley, PA

- Reservoir: 218,000 ac-ft; 5,700 acres

- Hydropower:  44 MW

- Project Objective:  Reduce hydrogen sulfide odors at powerhouse release.

- Customer:  PPL Generation

- Aeration System:  Line Diffuser

- - 50 HP air compressor

- - One 3,000-foot line diffuser located in old river channel

- - 200-scfm air delivery capacity

- Diffuser installed by Mobley Engineering, Inc.: 2004

- Norris Reservoir

- Located near Norris, Tennessee

- Reservoir:  2,040,000 ac-ft; 34,000 acres

- Hydropower:  two 60-MW hydroturbine units; 9,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 1 mg/L of dissolved oxygen

- Customer: Tennessee Valley Authority, Lake Improvement Plan II

- Aeration System: Line Diffusers

- - 15,000-gallon bulk liquid oxygen storage facility

- - Three line diffusers arranged in the forebay along old riverbed

- - Total of 12,000 feet of line diffuser deployed in reservoir

- Installed by TVA:  2004

- Nottely Reservoir

- Hydropower reservoir

- Located near Blairsville, Georgia

- Reservoir:  174,000 ac-ft; 4,000 acres

- Hydropower:  one 15 MW hydroturbine unit, 1,500 cfs

- Purpose for Aeration:  increase hydroturbine releases by 2 mg/L of dissolved oxygen

- Customer: Tennessee Valley Authority, Lake Improvement Plan II

- Aeration System: Line Diffusers

- - Two line diffusers arranged in the forebay along old riverbed

- - Total of 8,000 feet of line diffuser deployed in reservoir

- Installed by TVA:  2004

- Richard B. Russell Reservoir

- Located near Calhoun Falls, South Carolina

- Reservoir: 1,026,244 ac-ft, 26,500 acres

- Hydropower:  8 units, 600 MW total

- Project Objective:  Replace existing diffuser system.  Maintain 6 mg/L in releases.

- Customer:  US Army Corps of Engineers, Savannah District

- Aeration System:  Line Diffuser

- - Existing 40,000-gallon bulk liquid oxygen storage facility

- - Ten 4,000-foot line diffusers arranged in the forebay

- - 3,333-scfm (200 tons per day) oxygen delivery capacity

- Installed by: TVA, Draper & Associates, and Mobley Engineering Inc.: 2001

- Belews Creek Reservoir

- Fossil power cooling lake

- Located near Winston Salem, North Carolina

- Reservoir: 180,000 ac-ft, 4,100 acres

- Project Objective:  Reduce condenser cooling water temperatures to improve plant efficiency

- Customer:  Duke Power

- Air Plume Mixing System:  Line Diffuser

- - Compressed air fed from plant system

- - Eight bubble plumes saced at 60 feet over a 600-foot line diffusers arranged 1500 feet upstream of condenser cooling water intakes

- Installed by TVA, 1997

Buzzard Roost Hydroelectric Station

- Located near Greenwood, South Carolina

- Reservoir:  456,000 ac-ft

- Hydropower:  three 5-MW hydroturbine units; 3,675 cfs total turbine flow

- Purpose for Aeration:  Increase hydroturbine and minimum flow releases by

- 1 to 4 mg/L of dissolved oxygen

- Customer:  Duke Power Company

- Prime Contractor: Duke Engineering and Services, Charlotte, NC

- Aeration System:  Line Diffuser

- - 13,000-gallon bulk liquid oxygen storage facility

- - Three 3,000-foot line diffusers arranged in the intake channel and forebay

- - Total of 9,000 feet of line diffuser deployed in reservoir

- - 370 scfm or 22-tons/day oxygen system capacity

- Installated by TVA: 1997

Watts Bar Reservoir

- Located near Spring City, Tennessee

- Reservoir:  1,010,000 ac-ft; 39,000 acres

- Hydropower:  five 30-MW hydroturbine units; 45,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 1 mg/L of dissolved oxygen

- Customer: Tennesse Valley Authority, Lake Improvement Plan

- Aeration System: Line Diffusers

- - 15,000-gallon bulk liquid oxygen storage facility

- - Four line diffusers arranged in the forebay along old riverbed

- - Total of 24,000 feet of line diffuser deployed in reservoir

- - 900 scfm or 50-tons/day oxygen system capacity

- Installed by TVA:  1996

Hiwassee Reservoir

- Located near Murphy, North Carolina

- Reservoir:  422,000 ac-ft; 6,100 acres

- Hydropower:  two 60-MW hydroturbine units; 8,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 1.5 mg/L of dissolved

- oxygen

- Customer:  TVA Lake Improvement Plan

- Aeration System:  Line Diffuser

- - 15-ton oxygen per day pressure swing adsorption system 1997

- - 250-scfm PSA oxygen system capacity

- - Three line diffusers arranged in the forebay along old riverbed

- - Total of 12,600 feet of line diffuser deployed in reservoir

- - PSA system replaced with LOx tank and vaporizors 1999

- Line Diffuser Installation by TVA:  1995

Fort Loudoun Reservoir

- Located near Lenoir City, Tennessee

- Reservoir:  363,000 ac-ft; 14,600 acres

- Hydropower:  four 35-MW hydroturbine units; 28,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 1 mg/L of dissolved oxygen

- Customer:  TVA Lake Improvement Plan

- Aeration System:  Line Diffusers

- - 11,000-gallon bulk liquid oxygen storage facility

- - Two line diffusers arranged in the forebay along old riverbed

- - Total of 13,000 feet of line diffuser deployed in reservoir

- - 600 scfm or 36-tons/day oxygen system capacity

- Installed by TVA:  1995

Cherokee Reservoir

- Located near Jefferson City, Tennessee

- Reservoir:  1,481,000 ac-ft; 30,300 acres

- Hydropower:  four 30-MW hydroturbine units; 17,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 2 mg/L of dissolved oxygen

- Customer:  TVA Lake Improvement Plan

- Aeration System:  Line Diffusers

- - 20,000-gallon bulk liquid oxygen storage facility

- - Sixteen line diffusers arranged in the forebay along old riverbed

- - Total of 48,000 feet of line diffuser deployed in reservoir

- - 2,600 scfm or 150-tons/day oxygen system capacity

- Installed by TVA:  1994, 1995

Blue Ridge Reservoir

- Located near Blue Ridge, Georgia

- Reservoir:  193,000 ac-ft; 3,300 acres

- Hydropower:  one 22-MW hydroturbine unit; 1,800 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 3 mg/L of dissolved oxygen

- Customer:  TVA Lake Improvement Plan

- Aeration System:  Line Diffusers

- - 9,000 gallon bulk liquid oxygen storage facility

- - Four 1,800-foot long line diffusers arranged in the forebay

- - Total of 7,200 feet of line diffuser deployed in reservoir

- - 400 scfm or 24-tons/day oxygen system capacity

- Installed by TVA:  1994

Douglas Reservoir

- Located near Sevierville, Tennessee

- Reservoir:  1,408,000 ac-ft; 30,400 acres

- Hydropower:  four 30-MW hydroturbine units; 16,000 cfs total turbine flow

- Purpose for Aeration:  increase hydroturbine releases by 2 mg/L of dissolved oxygen

- Customer:  TVA Lake Improvement Plan

- Aeration System:  Diffuser Frames and Line Diffusers

- - 20,000-gallon bulk liquid oxygen storage facility

- - 16 porous hose diffuser frames arranged in the forebay

- - 16,000 feet of line diffuser located upstream of the frames

- - 1,800 scfm or 110-tons/day oxygen system capacity

- Installed by TVA:  1993, line diffuser additions by TVA:  1995, 1996